The Instruction Manual of Dmtool V2.x.xx.1

Version 1.1

V1 rwo - addition

10/21/2003

Compal Communications Inc.

Revisions

| Version | Primary Author(s) | Description of Version | Date Completed |
| --- | --- | --- | --- |
| 1.1 | Folk Chuang | V2.x.xx.1 | 10/30/2003 |

E365 Files Needed For Software Upgrade for Australia and New Zealand (to upgrade to software version 1.13)

Flash (use all these) If choosing to do individually then flash in top to bottom order

T66.1.0.13_L30.mot (main Flash file)

LP.T66.1.0.13.all.mot (Language Pack)

RSPK.T66.63.60.03.mot (Resource Pack)

Flex (choose 1 of the appropriate Network Flex files)

Flex.T66.60.40.02_Vod.mot (Vodafone AUS WAP GPRS settings)

Flex.T66.60.43.02_Nz.mot  (Vodafone NZ WAP GPRS settings)

Flex.T66.60.41.02_Tel.mot (Telstra WAP GPRS settings)

Flex.T66.60.42.02_Opt.mot (Optus WAP GPRS settings)

Rwo – v1 – Aus and NZ addition

Contents

Revisions	I

Contents	II

1.	Introduction	1

2.	Procedure	2

# Introduction

The purpose of this document is to describe the manual of the Dmtool V2.x.xx.1

# Procedure

- Open Dmtool_V2.x.xx.1.exe

- Under the Project Settings,

- The Tool is for E365.

- Download path is Rs232

- User can select COM port (COM1, COM2, …).

- Select the “All Mode” Check-Box  Dmtool will update the BootCode Region (flash block 0) when the user download the MainCode to our handset. (Default setting is the “Skip Mode”, this will not update the BootCode Region).

- Select the “Turbo DL” Check-Box  Use the Turbo Download (make sure the PC is equipped with Turbo-Card). (Default setting is the “Normal Rs232”, it means that use general PC COM port).

5.Click the “Add” Icon  To add a select-download-mot-file to the download-list.
Click the “Remove” Icon  To remove one select-download-mot-file from the download-list.
Click the “Clear” Icon  To clear all download-mot-file from the download-list.

Click the “Add” Icon will pop-up a Browse-Window which used to select the download-file.

- Click the “Execute” Icon  Start the download process.
Click the “Abort” Icon  Abort the download process. 
Click the “Close” Icon  Abort the download process and close the execution.

- Add mot file to the tool and press Execute button. After that press the power key in handset while the Interface showed as below.

7. While Downloading the Dmtool will show flash yellow light.

7.After download process completed, Dmtool will compare the target’s sector information data with the binary data in PC RAM buffer. If the data in these two buffers are identical, it means the download process complete and no any download error occurs. The Dmtool will show a Green Light  in the UI.

8.If any of difference found between these two buffers, the Dmtool will inform us the difference found in which block of the code.